# Supplementary Material
Excitation-Coherent Telemetry Analysis for Predictive OBD-II Connector Wear Detection: A Physics-Informed Study on Real In-Vehicle Network Data

P. N. Bhargav Teja, Lanka Sree Chathurya, K. Arun Reddy, Ragavan K

This document complements the article. Every number below is generated by python -m ocwd.experiments.supplement from the result files produced by the released code.

# Datasets and cleaning

All healthy telemetry is real and unmodified. Sessions are contiguous recordings after splitting at backward time steps or gaps >5 s and discarding sessions shorter than 120 s. CANmodes timestamps are repaired for an unpadded millisecond field, a seconds field that lags the millisecond counter after each wrap, and locale thousands separators; rows with non-hexadecimal identifiers are dropped. The four HCRL attack captures share one 482-s benign block, which is kept once; attack segments are excluded. From can-train-and-test only the attack-free drives are used. VED trips shorter than 150 s or with fewer than 20 engine- and vehicle-speed samples are excluded, and at most 40 trips per vehicle are used in the detection corpus.

TABLE S1. Sessions per vehicle after cleaning.

| Vehicle | Corpus | Sessions | Hours | Frames/responses | IDs |
| --- | --- | --- | --- | --- | --- |
| Ford Fiesta | sampling | 18 | 3.35 | 781 310 | 39 |
| GM Cruze | sampling | 34 | 5.23 | 1 261 129 | 79 |
| GM Impala | full-bus | 4 | 1.03 | 5 362 001 | 51 |
| GM Silverado | full-bus | 4 | 0.49 | 4 568 357 | 98 |
| GM Traverse | full-bus | 4 | 0.33 | 2 407 121 | 62 |
| KIA Soul | full-bus | 2 | 0.27 | 1 879 939 | 27 |
| MIRGU car | full-bus | 1 | 0.05 | 368 296 | 56 |
| Subaru Forester | full-bus | 4 | 0.50 | 2 873 382 | 54 |
| VW Gol | sampling | 11 | 2.07 | 444 502 | 40 |
| Ford Fiesta | polling | 11 | 2.71 | 57 613 | – |
| GM Cruze | polling | 27 | 3.61 | 72 908 | – |
| VW Gol | polling | 24 | 2.71 | 23 664 | – |

# Excitation decoders

Engine and vehicle speed are decoded only from frames or responses that reached the tool. Values outside 0–8000 rpm and 0–250 km/h are discarded. Polling sessions use PIDs 0x0C and 0x0D.

TABLE S2. Engine-speed and vehicle-speed decoders (identifier, first byte, byte order, scale).

| Vehicle | Engine speed (rpm) | Vehicle speed (km/h) |
| --- | --- | --- |
| GM Cruze | 0x0C9, byte 1, be, ×0.25 | 0x3E9, byte 0, be, ×0.01609 |
| VW Gol | 0x280, byte 2, le, ×0.25 | 0x1A0, byte 2, le, ×0.005 |
| KIA Soul | 0x316, byte 2, le, ×0.25 | 0x316, byte 6, u8, ×1 |
| GM Impala | 0x0C9, byte 1, be, ×0.25 | 0x3E9, byte 0, be, ×0.01609 |
| GM Traverse | 0x0C9, byte 1, be, ×0.25 | 0x3E9, byte 0, be, ×0.01609 |
| GM Silverado | 0x0C9, byte 1, be, ×0.25 | 0x3E9, byte 0, be, ×0.01609 |
| Subaru Forester | 0x140, byte 2, le14, ×1 | 0x0D1, byte 0, le, ×0.05625 |
| Ford Fiesta | 0x201, byte 0, be, ×1 | 0x201, byte 4, be, ×0.01 |

# Feature definitions

P denotes the median inter-arrival interval of an identifier (passive) or of the polling loop.

TABLE S3. ECTA features (25 passive, 20 polling).

| Tool | Family | Feature | Definition |
| --- | --- | --- | --- |
| passive | loss | rate | frames received per second |
| passive | loss | n_ids | distinct identifiers in the window |
| passive | loss | miss_ratio | missing frames / expected frames over periodic identifiers |
| passive | loss | gap_events_pm | gaps containing ≥1 missing frame, per minute |
| passive | loss | miss_pm | missing frames per minute |
| passive | loss | max_silence | longest all-identifier silence (s) |
| passive | loss | silences_pm | all-identifier silences per minute |
| passive | structure | late_ratio | share of intervals in 1.08P,1.5P without a missing frame (retransmission) |
| passive | structure | bunch_ratio | share of intervals below 0.5P (backlog release) |
| passive | structure | jitter_mad | median Δ/P−1 of regular intervals |
| passive | structure | sel_chi2 | log1+χ2/dof of per-ID losses vs. traffic-proportional losses |
| passive | structure | sel_gini | Gini coefficient of per-ID loss rates |
| passive | structure | sel_frac_ids | share of identifiers with any loss |
| passive | structure | load_at_gap | log ratio of local frame density at losses to window mean |
| passive | structure | load_corr | Spearman correlation of per-second losses and load |
| passive | coherence | coh_rho | Spearman ρ(loss events, v) |
| passive | coherence | coh_beta | Poisson GLM slope β |
| passive | coherence | coh_z | Wald statistic zβ |
| passive | coherence | coh_hilo | log rate ratio above/below median excitation |
| passive | coherence | coh2_rho | as coh_rho, strong events only |
| passive | coherence | coh2_beta | as coh_beta, strong events only |
| passive | coherence | coh2_z | as coh_z, strong events only |
| passive | coherence | coh2_hilo | as coh_hilo, strong events only |
| passive | coherence | exc_mean | mean excitation proxy v |
| passive | coherence | exc_avail | 1 if an excitation decoder exists |
| polling | loss | txn_rate | responses per second |
| polling | loss | med_dt | median inter-response interval (s) |
| polling | loss | gap_ratio | share of intervals above median + 0.12 s (timeout-like) |
| polling | loss | gaps_pm | timeout-like gaps per minute |
| polling | loss | excess_frac | excess time in timeout-like gaps / window |
| polling | loss | long_gaps_pm | gaps >0.8 s per minute (re-initialisation) |
| polling | loss | max_gap | longest inter-response gap (s) |
| polling | loss | pid_miss | missing PID cycles / expected cycles |
| polling | structure | jitter_mad | median Δ/P−1 of regular intervals |
| polling | structure | gap_dur_med | median excess duration of timeout-like gaps (s) |
| polling | structure | gap_dur_cv | coefficient of variation of those durations |
| polling | coherence | coh_rho | Spearman ρ(loss events, v) |
| polling | coherence | coh_beta | Poisson GLM slope β |
| polling | coherence | coh_z | Wald statistic zβ |
| polling | coherence | coh_hilo | log rate ratio above/below median excitation |
| polling | coherence | coh2_rho | as coh_rho, strong events only |
| polling | coherence | coh2_beta | as coh_beta, strong events only |
| polling | coherence | coh2_z | as coh_z, strong events only |
| polling | coherence | coh2_hilo | as coh_hilo, strong events only |
| polling | coherence | exc_mean | mean excitation proxy v |

# Full detection results

TABLE S4. Full-bus passive: all methods (6468 windows, 6 vehicles, 6 folds). All = wear vs. all; Inc./Mod./Sev. vs. healthy; ECU/EMI/Ovf. = moderate+severe wear vs. that confounder.

| Method | AUROC | AUPRC | TPR@1% | Inc. | Mod. | Sev. | ECU | EMI | Ovf. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ECTA (proposed) | 0.880 | 0.880 | 0.554 | 0.555 | 0.891 | 0.995 | 0.949 | 0.979 | 0.977 |
| GBM w/o coherence | 0.878 | 0.877 | 0.560 | 0.549 | 0.881 | 0.996 | 0.945 | 0.978 | 0.977 |
| GBM loss only | 0.768 | 0.763 | 0.308 | 0.540 | 0.761 | 0.955 | 0.865 | 0.935 | 0.778 |
| GBM loss+coherence | 0.812 | 0.799 | 0.315 | – | – | – | – | – | – |
| Isolation Forest | 0.622 | 0.569 | 0.044 | 0.584 | 0.939 | 0.992 | 0.894 | 0.762 | 0.460 |
| One-class SVM | 0.596 | 0.528 | 0.012 | 0.544 | 0.830 | 0.958 | 0.828 | 0.672 | 0.492 |
| Loss-rate threshold | 0.653 | 0.651 | 0.280 | 0.592 | 0.963 | 0.996 | 0.914 | 0.830 | 0.546 |
| U-code timeout rule | 0.587 | 0.620 | 0.285 | 0.500 | 0.539 | 0.922 | 0.728 | 0.529 | 0.718 |
| Coherence test (no training) | 0.594 | 0.589 | 0.161 | 0.542 | 0.684 | 0.692 | 0.659 | 0.638 | 0.597 |
| 1D-CNN (supervised) | 0.646 | 0.669 | 0.283 | 0.500 | 0.522 | 0.895 | 0.733 | 0.712 | 0.704 |
| LSTM autoencoder | 0.631 | 0.665 | 0.291 | 0.500 | 0.524 | 0.907 | 0.715 | 0.677 | 0.715 |

TABLE S5. Sampling passive: all methods (12684 windows, 3 vehicles, 3 folds). All = wear vs. all; Inc./Mod./Sev. vs. healthy; ECU/EMI/Ovf. = moderate+severe wear vs. that confounder.

| Method | AUROC | AUPRC | TPR@1% | Inc. | Mod. | Sev. | ECU | EMI | Ovf. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ECTA (proposed) | 0.675 | 0.702 | 0.326 | 0.501 | 0.556 | 0.957 | 0.759 | 0.766 | 0.756 |
| GBM w/o coherence | 0.674 | 0.701 | 0.316 | 0.500 | 0.554 | 0.955 | 0.758 | 0.764 | 0.755 |
| GBM loss only | 0.673 | 0.697 | 0.287 | 0.500 | 0.551 | 0.952 | 0.755 | 0.764 | 0.752 |
| GBM loss+coherence | 0.675 | 0.701 | 0.318 | – | – | – | – | – | – |
| Isolation Forest | 0.637 | 0.614 | 0.110 | 0.500 | 0.530 | 0.883 | 0.706 | 0.704 | 0.707 |
| One-class SVM | 0.658 | 0.661 | 0.170 | 0.500 | 0.542 | 0.931 | 0.737 | 0.737 | 0.737 |
| Loss-rate threshold | 0.667 | 0.619 | 0.340 | 0.500 | 0.551 | 0.948 | 0.750 | 0.750 | 0.750 |
| U-code timeout rule | 0.668 | 0.667 | 0.124 | 0.501 | 0.587 | 0.952 | 0.763 | 0.741 | 0.770 |
| Coherence test (no training) | 0.510 | 0.433 | 0.010 | 0.500 | 0.499 | 0.532 | 0.516 | 0.515 | 0.516 |
| 1D-CNN (supervised) | 0.677 | 0.700 | 0.345 | 0.500 | 0.569 | 0.964 | 0.764 | 0.766 | 0.767 |
| LSTM autoencoder | 0.651 | 0.625 | 0.018 | 0.500 | 0.534 | 0.920 | 0.726 | 0.727 | 0.727 |

TABLE S6. OBD-II polling: all methods (8631 windows, 3 vehicles, 3 folds). All = wear vs. all; Inc./Mod./Sev. vs. healthy; ECU/EMI/Ovf. = moderate+severe wear vs. that confounder.

| Method | AUROC | AUPRC | TPR@1% | Inc. | Mod. | Sev. | ECU | EMI | Ovf. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ECTA (proposed) | 0.723 | 0.727 | 0.297 | 0.500 | 0.528 | 0.891 | 0.753 | 0.878 | 0.762 |
| GBM w/o coherence | 0.686 | 0.705 | 0.277 | 0.500 | 0.524 | 0.898 | 0.761 | 0.741 | 0.772 |
| GBM loss only | 0.670 | 0.682 | 0.250 | 0.500 | 0.527 | 0.845 | 0.740 | 0.719 | 0.753 |
| GBM loss+coherence | 0.712 | 0.708 | 0.270 | – | – | – | – | – | – |
| Isolation Forest | 0.599 | 0.549 | 0.068 | 0.500 | 0.523 | 0.821 | 0.628 | 0.670 | 0.656 |
| One-class SVM | 0.603 | 0.564 | 0.058 | 0.500 | 0.511 | 0.864 | 0.665 | 0.618 | 0.690 |
| Loss-rate threshold | 0.579 | 0.583 | 0.141 | 0.500 | 0.520 | 0.893 | 0.598 | 0.703 | 0.607 |
| U-code timeout rule | 0.609 | 0.646 | 0.209 | 0.500 | 0.575 | 0.962 | 0.690 | 0.701 | 0.699 |
| Coherence test (no training) | 0.485 | 0.455 | 0.026 | 0.500 | 0.498 | 0.413 | 0.479 | 0.478 | 0.461 |
| 1D-CNN (supervised) | 0.673 | 0.692 | 0.272 | 0.501 | 0.512 | 0.882 | 0.732 | 0.736 | 0.750 |
| LSTM autoencoder | 0.548 | 0.503 | 0.061 | 0.500 | 0.551 | 0.795 | 0.596 | 0.611 | 0.596 |

TABLE S7. VED fleet: all methods (236817 windows, 381 vehicles, 5 folds). All = wear vs. all; Inc./Mod./Sev. vs. healthy; ECU/EMI/Ovf. = moderate+severe wear vs. that confounder.

| Method | AUROC | AUPRC | TPR@1% | Inc. | Mod. | Sev. | ECU | EMI | Ovf. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ECTA (proposed) | 0.775 | 0.737 | 0.235 | 0.500 | 0.493 | 0.851 | 0.750 | 0.947 | 0.856 |
| GBM w/o coherence | 0.704 | 0.678 | 0.190 | 0.500 | 0.497 | 0.845 | 0.749 | 0.737 | 0.825 |
| GBM loss only | 0.672 | 0.643 | 0.148 | 0.500 | 0.501 | 0.806 | 0.714 | 0.708 | 0.771 |
| GBM loss+coherence | 0.757 | 0.715 | 0.197 | – | – | – | – | – | – |
| Isolation Forest | 0.551 | 0.496 | 0.020 | 0.500 | 0.494 | 0.739 | 0.595 | 0.570 | 0.584 |
| One-class SVM | 0.541 | 0.489 | 0.029 | 0.500 | 0.488 | 0.682 | 0.592 | 0.504 | 0.598 |
| Loss-rate threshold | 0.567 | 0.542 | 0.073 | 0.500 | 0.508 | 0.813 | 0.617 | 0.657 | 0.560 |
| U-code timeout rule | 0.557 | 0.467 | 0.006 | 0.500 | 0.535 | 0.796 | 0.615 | 0.654 | 0.533 |
| Coherence test (no training) | 0.509 | 0.442 | 0.011 | 0.500 | 0.496 | 0.440 | 0.492 | 0.606 | 0.423 |
| 1D-CNN (supervised) | 0.701 | 0.698 | 0.249 | 0.500 | 0.539 | 0.911 | 0.756 | 0.734 | 0.841 |
| LSTM autoencoder | 0.554 | 0.480 | 0.010 | 0.500 | 0.502 | 0.703 | 0.592 | 0.604 | 0.556 |

# Per-vehicle results

TABLE S8. ECTA per held-out vehicle (leave-one-vehicle-out).

| Corpus | Held-out vehicle | Windows | All | Moderate | Severe |
| --- | --- | --- | --- | --- | --- |
| Full-bus passive | GM Impala | 2541 | 0.882 | 0.900 | 0.999 |
| Full-bus passive | GM Silverado | 1176 | 0.886 | 0.863 | 0.995 |
| Full-bus passive | GM Traverse | 798 | 0.921 | 0.951 | 0.982 |
| Full-bus passive | KIA Soul | 651 | 0.869 | 0.946 | 0.978 |
| Full-bus passive | MIRGU car | 126 | 0.904 | 0.898 | 1.000 |
| Full-bus passive | Subaru Forester | 1176 | 0.910 | 0.982 | 0.994 |
| Sampling passive | Ford Fiesta | 3990 | 0.681 | 0.566 | 0.954 |
| Sampling passive | GM Cruze | 6216 | 0.684 | 0.567 | 0.963 |
| Sampling passive | VW Gol | 2478 | 0.672 | 0.533 | 0.961 |
| OBD-II polling | Ford Fiesta | 2898 | 0.786 | 0.559 | 0.932 |
| OBD-II polling | GM Cruze | 3360 | 0.782 | 0.546 | 0.957 |
| OBD-II polling | VW Gol | 2373 | 0.673 | 0.529 | 0.929 |

# Robustness to mis-specified physics

Trained on the nominal model, tested on held-out vehicles generated with each perturbation.

TABLE S9. Robustness, Full-bus passive (AUROC vs. healthy and confounders).

| Test physics | All | Incipient | Moderate | Severe |
| --- | --- | --- | --- | --- |
| nominal | 0.901 | 0.766 | 0.940 | 0.997 |
| sigma=0.5 | 0.851 | 0.701 | 0.858 | 0.995 |
| sigma=1.2 | 0.927 | 0.833 | 0.956 | 0.994 |
| d0 x0.25 | 0.864 | 0.711 | 0.895 | 0.986 |
| d0 x4 | 0.916 | 0.788 | 0.966 | 0.995 |
| nu0 x0.5 | 0.875 | 0.717 | 0.909 | 0.998 |
| nu0 x2 | 0.905 | 0.771 | 0.951 | 0.994 |
| R_th=3 | 0.948 | 0.862 | 0.989 | 0.995 |
| R_th=15 | 0.837 | 0.693 | 0.837 | 0.982 |
| p_power=0.1 | 0.901 | 0.772 | 0.937 | 0.994 |
| p_power=0.6 | 0.886 | 0.752 | 0.916 | 0.992 |
| holdup=0.5ms | 0.906 | 0.766 | 0.957 | 0.996 |
| holdup=10ms | 0.901 | 0.766 | 0.940 | 0.996 |
| road noise 0 | 0.888 | 0.741 | 0.926 | 0.996 |
| road noise x2 | 0.893 | 0.749 | 0.932 | 0.999 |
| engine-only excitation | 0.893 | 0.737 | 0.941 | 1.000 |
| road-only excitation | 0.875 | 0.739 | 0.918 | 0.968 |

TABLE S10. Robustness, VED fleet (AUROC vs. healthy and confounders).

| Test physics | All | Incipient | Moderate | Severe |
| --- | --- | --- | --- | --- |
| nominal | 0.748 | 0.686 | 0.670 | 0.887 |
| sigma=0.5 | 0.754 | 0.688 | 0.679 | 0.895 |
| sigma=1.2 | 0.745 | 0.685 | 0.667 | 0.882 |
| d0 x0.25 | 0.660 | 0.667 | 0.663 | 0.649 |
| d0 x4 | 0.774 | 0.661 | 0.728 | 0.934 |
| nu0 x0.5 | 0.731 | 0.682 | 0.672 | 0.838 |
| nu0 x2 | 0.761 | 0.688 | 0.672 | 0.922 |
| R_th=3 | 0.792 | 0.677 | 0.731 | 0.966 |
| R_th=15 | 0.688 | 0.671 | 0.670 | 0.722 |
| p_power=0.1 | 0.713 | 0.682 | 0.670 | 0.788 |
| p_power=0.6 | 0.759 | 0.681 | 0.663 | 0.934 |
| holdup=0.5ms | 0.774 | 0.660 | 0.728 | 0.935 |
| holdup=10ms | 0.653 | 0.670 | 0.666 | 0.623 |
| road noise 0 | 0.754 | 0.694 | 0.680 | 0.888 |
| road noise x2 | 0.746 | 0.684 | 0.666 | 0.887 |
| engine-only excitation | 0.745 | 0.690 | 0.675 | 0.870 |
| road-only excitation | 0.746 | 0.684 | 0.669 | 0.885 |

# Prognosis and multi-trip pooling

TABLE S11. VED vehicle-level detection after pooling K trips (left: vs. all; right: vs. healthy).

| K | Vehicles | Inc. | Mod. | Sev. | Inc. | Mod. | Sev. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 381 | 0.718 | 0.704 | 0.907 | 0.500 | 0.483 | 0.850 |
| 2 | 368 | 0.759 | 0.753 | 0.965 | 0.487 | 0.478 | 0.934 |
| 3 | 354 | 0.782 | 0.767 | 0.987 | 0.484 | 0.461 | 0.971 |
| 5 | 341 | 0.809 | 0.802 | 0.995 | 0.502 | 0.489 | 0.989 |
| 10 | 320 | 0.830 | 0.817 | 1.000 | 0.500 | 0.470 | 0.999 |
| 20 | 272 | 0.844 | 0.835 | 1.000 | 0.503 | 0.483 | 1.000 |

TABLE S12. Prognosis, failure at 6 Ω (110 of 166 vehicles fail; health-index R2 = 0.86).

| Method | MAE (d) | Median AE (d) | α=0.3 acc. % | Timely % | Prem. % | Missed % | Lead (d) | FA % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| proposed | 71 | 51 | 24 | 13 | 29 | 58 | 0.0 | 16.1 |
| raw_hi | 72 | 54 | 25 | 5 | 7 | 87 | 0.0 | 1.8 |
| conditional | 64 | 44 | 26 | 7 | 7 | 85 | 0.0 | 0.0 |
| fleet | 66 | 44 | 19 | 36 | 28 | 35 | 18.7 | 60.7 |
| linear | – | – | 2 | 8 | 10 | 82 | 0.0 | 3.6 |

TABLE S13. Prognosis, failure at 20 Ω (100 of 166 vehicles fail; health-index R2 = 0.86).

| Method | MAE (d) | Median AE (d) | α=0.3 acc. % | Timely % | Prem. % | Missed % | Lead (d) | FA % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| proposed | 71 | 51 | 24 | 45 | 28 | 27 | 4.0 | 12.1 |
| raw_hi | 79 | 61 | 26 | 2 | 6 | 92 | 0.0 | 1.5 |
| conditional | 67 | 46 | 26 | 5 | 10 | 85 | 0.0 | 0.0 |
| fleet | 69 | 47 | 18 | 36 | 34 | 30 | 25.1 | 60.6 |
| linear | – | – | 3 | 88 | 5 | 7 | 6.4 | 1.5 |

# Cross-model generalization

The test faults are generated by a structurally different process: a two-state (Gilbert–Elliott) contact alternating between quiet periods and chatter episodes (mean 0.1 s, 100 interruptions/s), with exponentially distributed durations and a brown-out triggered when the cumulative open time of a supply-pin episode exceeds the hold-up time. Expected interruption rate and mean duration equal those of the nominal Poisson model; temporal clustering, duration law and brown-out mechanism differ. The detector is trained only on the nominal model.

TABLE S14. Train on the nominal model, test on the nominal or the Markov model (vehicle-disjoint).

| Corpus | Train → test | All | Incipient | Moderate | Severe |
| --- | --- | --- | --- | --- | --- |
| Full-bus passive | nominal → nominal | 0.880 | 0.555 | 0.891 | 0.995 |
| Full-bus passive | nominal → Markov | 0.856 | 0.528 | 0.848 | 0.999 |
| OBD-II polling | nominal → nominal | 0.723 | 0.500 | 0.528 | 0.891 |
| OBD-II polling | nominal → Markov | 0.779 | 0.501 | 0.675 | 0.941 |
| VED fleet | nominal → nominal | 0.771 | 0.500 | 0.490 | 0.842 |
| VED fleet | nominal → Markov | 0.813 | 0.499 | 0.577 | 0.950 |

# Operating points and calibration

The threshold is set to reach the stated detection rate for the given wear stage; false alarms are counted on the real healthy windows of held-out vehicles.

TABLE S15. False alarms at fixed detection rates.

| Corpus | Stage | Detection | FPR healthy | False alarms / h | FPR confounders |
| --- | --- | --- | --- | --- | --- |
| Full-bus passive | moderate | 80% | 11.0% | 13.25 | 6.6% |
| Full-bus passive | moderate | 90% | 52.6% | 63.12 | 25.3% |
| Full-bus passive | moderate | 95% | 66.6% | 79.87 | 34.1% |
| Full-bus passive | severe | 80% | 0.0% | 0.00 | 0.0% |
| Full-bus passive | severe | 90% | 0.0% | 0.00 | 0.1% |
| Full-bus passive | severe | 95% | 0.3% | 0.39 | 0.4% |
| Sampling passive | moderate | 80% | 76.5% | 45.89 | 75.6% |
| Sampling passive | moderate | 90% | 88.1% | 52.85 | 86.6% |
| Sampling passive | moderate | 95% | 94.2% | 56.52 | 92.6% |
| Sampling passive | severe | 80% | 0.3% | 0.20 | 0.3% |
| Sampling passive | severe | 90% | 2.6% | 1.59 | 2.6% |
| Sampling passive | severe | 95% | 44.2% | 26.52 | 43.0% |
| OBD-II polling | moderate | 80% | 79.3% | 23.80 | 65.3% |
| OBD-II polling | moderate | 90% | 89.5% | 26.86 | 77.4% |
| OBD-II polling | moderate | 95% | 94.6% | 28.39 | 83.7% |
| OBD-II polling | severe | 80% | 1.2% | 0.36 | 1.0% |
| OBD-II polling | severe | 90% | 66.2% | 19.85 | 41.8% |
| OBD-II polling | severe | 95% | 74.2% | 22.26 | 60.1% |
| VED fleet | moderate | 80% | 83.3% | 7.10 | 35.7% |
| VED fleet | moderate | 90% | 92.3% | 7.86 | 45.1% |
| VED fleet | moderate | 95% | 96.2% | 8.20 | 52.8% |
| VED fleet | severe | 80% | 19.7% | 1.68 | 8.4% |
| VED fleet | severe | 90% | 78.9% | 6.73 | 32.7% |
| VED fleet | severe | 95% | 93.2% | 7.94 | 46.7% |

TABLE S16. Calibration of the ECTA wear probability (wear vs. all).

| Corpus | Brier | ECE |
| --- | --- | --- |
| Full-bus passive | 0.145 | 0.098 |
| Sampling passive | 0.312 | 0.276 |
| OBD-II polling | 0.237 | 0.180 |
| VED fleet | 0.185 | 0.052 |

# Window length

TABLE S17. Full-bus detection vs. window length (one seed, leave-one-vehicle-out).

| Window | Windows | All | Incipient | Moderate | Severe |
| --- | --- | --- | --- | --- | --- |
| 10s | 6629 | 0.824 | 0.543 | 0.843 | 0.983 |
| 30s | 2156 | 0.879 | 0.565 | 0.895 | 0.995 |
| 60s | 1050 | 0.911 | 0.643 | 0.930 | 0.999 |

# Observability law

TABLE S18. Inputs and outputs of the observability analysis (Spearman ρ = 0.93).

| Corpus | Stage | R (Ω) | Unit rate (1/s) | A | σB | SNR | Best AUROC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Full-bus passive | incipient | 0.67 | 1613.5 | 0.206 | 49.7 | 0.00414 | 0.592 |
| Full-bus passive | moderate | 3.00 | 1613.5 | 44.8 | 49.7 | 0.9 | 0.963 |
| Full-bus passive | severe | 13.42 | 1613.5 | 6.95e+03 | 49.7 | 140 | 0.996 |
| Sampling passive | incipient | 0.67 | 65.6 | 6.75e-07 | 0.0909 | 7.43e-06 | 0.501 |
| Sampling passive | moderate | 3.00 | 65.6 | 0.0282 | 0.0909 | 0.31 | 0.587 |
| Sampling passive | severe | 13.42 | 65.6 | 9.31 | 0.0909 | 102 | 0.964 |
| OBD-II polling | incipient | 0.67 | 5.6 | 0.0108 | 33.9 | 0.000319 | 0.501 |
| OBD-II polling | moderate | 3.00 | 5.6 | 1.18 | 33.9 | 0.0348 | 0.575 |
| OBD-II polling | severe | 13.42 | 5.6 | 29.5 | 33.9 | 0.869 | 0.962 |
| VED fleet | incipient | 0.67 | 1.3 | 0.00785 | 54.5 | 0.000144 | 0.500 |
| VED fleet | moderate | 3.00 | 1.3 | 0.992 | 54.5 | 0.0182 | 0.539 |
| VED fleet | severe | 13.42 | 1.3 | 66.1 | 54.5 | 1.21 | 0.911 |

# Reproducibility

Software: Python 3.11, NumPy 2.4.6, pandas 3.0.6, SciPy 1.17.1, scikit-learn 1.9.1, LightGBM 4.7.0, PyTorch 2.14.1 (CPU). Hardware: 4-core CPU, no GPU. Complete pipeline: ocwd/get_data.sh then ocwd/run_all.sh (about 4–5 h). Fourteen unit tests (ocwd/tests) cover the timestamp repair, interruption statistics, loss mechanics, Poisson coherence test, Markov fault model and baselines. Random seeds: three injection seeds (0, 1, 2) for all detection corpora; per-session streams are derived from CRC32 of the session name, so results are independent of execution order.